Quick Start Guide: Custom BLE Bridge Plugin

**Plugin:** `com.mbrats01.ble_manager`

**Author:** MichaellaBratsa

**Date:** 2025-10-16

Description

Provide a simple and lightweight bridge for communication between Unity (on Android) and a Bluetooth Low Energy (BLE) device, such as the Arduino Nano 33 BLE.

Overview

This plugin consists of two main parts:

1. The Android Archive (.aar): A com.mbrats01.ble_manager.aar file containing the native Java/Kotlin code. This handles the low-level tasks of scanning, connecting, and receiving BLE data.

2.  The C# Script (JavaBridge.cs): A Unity script that acts as a bridge, allowing C# code to issue commands to the native plugin and receive responses.

This guide demonstrates how to set up a minimal project to send a simple message from an Arduino to Unity.

Step 1: Setting Up the Arduino (The Transmitter)

In this step, we will program the Arduino to advertise itself with a specific name and send a simple, incrementing message every second once a connection is established.

1.  Hardware: Use an Arduino Nano 33 BLE

2.  Arduino IDE: Ensure you have the ArduinoBLE library installed.

3.  Code: Upload the following sample sketch to your Arduino.

SampleArduinoSketch.cpp

#include <ArduinoBLE.h>

//Define unique UUIDs for the service and characteristic

BLEService customService("6E400001-B5A3-F393-E0A9-E50E24DCCA9E");

BLEStringCharacteristic messageCharacteristic(

" 6E400003-B5A3-F393-E0A9-E50E24DCCA9E", BLERead | BLENotify, 20);

int messageCounter = 0;

void setup() {

Serial.begin(9600);

while (!Serial);

if (!BLE.begin()) {

Serial.println("Starting BLE failed!");

while (1);

}

// Set the name that will appear during a scan

BLE.setLocalName("MyArduinoBLE");

BLE.setAdvertisedService(customService);

customService.addCharacteristic(messageCharacteristic);

BLE.addService(customService);

messageCharacteristic.writeValue("Ready");

BLE.advertise();

Serial.println("Arduino BLE Server is running...");

}

void loop() {

BLEDevice central = BLE.central();

if (central) {

Serial.print("Connected to central: ");

Serial.println(central.address());

while (central.connected()) {

// Create a message

String message = "Message #" + String(messageCounter);

// Add the newline character as a delimiter

message += '\n';

// Send the message

messageCharacteristic.writeValue(message);

Serial.print("Sent: ");

Serial.println(message);

messageCounter++;

delay(1000);  // Send every second

}

Serial.print("Disconnected from central: ");

Serial.println(central.address());

}

}

Step 2: Setting Up the Unity Project (The Receiver)

Step 2.1: Install the Plugin

1.  Open your Unity project.

2.  Create the folder Assets/Plugins/Android/ if it does not already exist.

3.  Place the BleBridge.aar file inside this folder. *This step is critical. Without it, the plugin will not be found.

Step 2.2: Create the Bridge

- Create a new C# Script named e.g. JavaBridge.cs and replace its contents with the code below.

JavaBridge.cs

using UnityEngine;

using UnityEngine.Android;

using TMPro;

public class JavaBridge : MonoBehaviour

{

public TextMeshProUGUI statusText;

public TextMeshProUGUI receivedDataText;

public GameObject connectButton;

private AndroidJavaObject bleManager;

private string targetDeviceAddress = null;

private string partialMessage = "";

void Start()

{

connectButton.SetActive(false);

statusText.text = "Bridge Initialized. Ready to Scan.";

receivedDataText.text = "--- Received Data ---";

// Initialize the native plugin

using (AndroidJavaClass managerClass = new AndroidJavaClass("com.mbrats01.ble_manager.BleManager"))

{

bleManager = managerClass.CallStatic<AndroidJavaObject>("getInstance", GetUnityActivity());

}

if (bleManager != null)

{

bleManager.Call("setCallback", new BleCallbackProxy(this));

}

else

{

statusText.text = "Error: BLE Plugin not found!";

Debug.LogError("bleManager is NULL. Make sure the .aar file is in Assets/Plugins/Android/");

}

}

// --- Public methods for UI Buttons ---

public void RequestBlePermissions()

{

var callbacks = new PermissionCallbacks();

int sdkInt = new AndroidJavaClass("android.os.Build$VERSION").GetStatic<int>("SDK_INT");

if (sdkInt >= 31) // Android 12+

{

Permission.RequestUserPermissions(new string[] {

"android.permission.BLUETOOTH_SCAN",

"android.permission.BLUETOOTH_CONNECT",

"android.permission.ACCESS_COARSE_LOCATION",

"android.permission.ACCESS_FINE_LOCATION"}, callbacks);

}

else

{

Permission.RequestUserPermissions(new string[]

{

"android.permission.ACCESS_FINE_LOCATION"

}, callbacks);

}

}

public void StartBleScan()

{

if (bleManager == null) return;

targetDeviceAddress = null;

connectButton.SetActive(false);

statusText.text = "Scanning for devices...";

bleManager.Call("startScan");

}

public void ConnectToDevice()

{

if (bleManager == null || string.IsNullOrEmpty(targetDeviceAddress)) return;

statusText.text = $"Connecting to {targetDeviceAddress}...";

bleManager.Call("connectToDevice", targetDeviceAddress);

}

// --- Callbacks invoked by the native plugin ---

public void OnStatusUpdate(string message)

{

UnityMainThreadDispatcher.Instance().Enqueue(() => { statusText.text = "Status: " + message; });

}

public void OnDeviceFound(string name, string address)

{

UnityMainThreadDispatcher.Instance().Enqueue(() => {

if (name != null && name.Contains("MyArduinoBLE"))

{

statusText.text = $"Device Found: {name}";

targetDeviceAddress = address;

connectButton.SetActive(true);

}

});

}

public void OnDataReceived(string message)

{

// Accumulate data to handle fragmentation

partialMessage += message;

int newlineIndex;

while ((newlineIndex = partialMessage.IndexOf('\n')) != -1)

{

string completeMessage = partialMessage.Substring(0, newlineIndex).Trim();

partialMessage = partialMessage.Substring(newlineIndex + 1);

if (!string.IsNullOrEmpty(completeMessage))

{

// Display the final message on the UI

UnityMainThreadDispatcher.Instance().Enqueue(() => {

Debug.Log("Complete Message Received: " + completeMessage);

receivedDataText.text = completeMessage;

});

}

}

}

// --- Helper method to get the Unity Activity ---

private AndroidJavaObject GetUnityActivity()

{

using (var unityPlayer = new AndroidJavaClass("com.unity3d.player.UnityPlayer"))

{

return unityPlayer.GetStatic<AndroidJavaObject>("currentActivity");

}

}

}

// --- Proxy Class for Callbacks ---

class BleCallbackProxy : AndroidJavaProxy

{

private readonly JavaBridge bridge;

public BleCallbackProxy(JavaBridge bridge) : base("com.mbrats01.ble_manager.BleCallback") { this.bridge = bridge; }

public void onStatusUpdate(string message) { bridge.OnStatusUpdate(message); }

public void onDeviceFound(string name, string address) { bridge.OnDeviceFound(name, address); }

public void onDataReceived(string message) { bridge.OnDataReceived(message); }

}

Step 3.3: Set Up the Scene in Unity

1.Create an Empty GameObject named e.g. BleBridgeObject and attach the JavaBridge.cs script to it.

2. Create a UI Canvas and add:

- A Button to request permissions (RequestBlePermissions).

- A Button to start the scan (StartBleScan).

- A Button to connect (ConnectToDevice). Initially, set it to be inactive.

- Two TextMeshPro - Text objects to display the status and received data.

3. Connect the UI elements to their corresponding public fields (statusText, receivedDataText, connectButton) in the JavaBridge.cs script via the Inspector.

4.  Configure the OnClick() events of the buttons in the Inspector to call the respective public methods of the JavaBridge script (RequestBlePermissions, StartBleScan, ConnectToDevice).

Step 3.4: Build and Run

1.  Go to File > Build Settings and switch the platform to Android.

2.  Build and Run on a physical Android device.

3.  Follow the flow:

Press the permissions button. You must allow to access the device’s location (“While using the app”) and to determine the relative position nearby devices (“Allow”)

Press the "Scan" button. The status text should update to "Device Found: MyArduinoBLE". The "Connect" button will become active.

*   Press the "Connect" button.

*   The receivedDataText field should now update every second with messages from the Arduino ("Message #1", "Message #2", etc.).

If all steps are completed correctly, you have successfully connected your Arduino to Unity using the custom BLE plugin!